Project 1: FIR Filter Design

# Introduction

In this project, you will design 256 tap FIR filter using high level synthesis tools. We provide baseline code and you must perform optimizations to increase the throughput, decrease the area, and reduce power/energy consumption. The end results is a report that provides the synthesizable code, describes the optimizations and their architecture, and a discussion on the effect that each optimization had on throughput, area and power/energy consumption.

# Preparation

Before you start this project, you must first do the first chapter of the tutorial which describes the high level synthesis tool chain: ug871-vivado-high-level-synthesis-tutorial.pdf located in Tutorial folder.

# Materials

You can find the following files in project1-fir.zip:

- fir.cpp  Implements to level function

- fir.h header file

- fir_test.cpp  test bench

- out.gold.dat  “Golden” output. When the testbench (from fir_test.cpp) is run through the file fir.cpp it should generate this result. If it does not, you did something wrong.

- project1-fir.pdf  project instructions: this file

- Tutorial folder

  - ug871-vivado-high-level-synthesis-tutorial.pdf  Tutorial for Vivado HLS.

  - ug871-design-files.zip Source files used for the tutorial

The fir.cpp source file contains fir() function that computes FIR. The function prototype is shown in Figure 1. This initial code will simulate correctly, i.e., the results should match the golden output.

Figure 1. Given FIR code.

# Project Goal

You should modify this code to generate at least four additional architectures. Your goal is to create architectures that provide tradeoffs between area and execution time. This will require you to rewrite the code and/or insert pragmas. You will submit separate files for each of the different architectures. You will write a report that details each of the different architectures.

You must set your target clock period to 10 ns, though you can report your throughput in terms of the estimated clock period.

# Optimization Hints

You must generate at least one architecture that utilizes all of these optimization hints. This architecture does not have to use all of the hints, e.g., you could generate five different architectures where each one performs one of these optimizations. You are free to use any additional optimizations. The only restriction is that your resulting code is correct, i.e., it matches the golden output.

Some possibilities for optimization include:

- Variable Bitwidths: The bitwidth of the variables provides a tradeoff between precision, area, and performance. It is possible to specify the exact size of each variable to the tools. Make sure that you do not effect the results, i.e., the output still matches the golden output.

- Removing Conditional Statments: If/else statements and other conditionals limit the possible parallelism and create additional area. If the code can be rewritten to remove them, it typically makes the resulting architecture smaller and faster.

- Pipelining: This increases the throughput at the cost of adding area (registers). The area and peformance can be varied by changing the initiation interval (II).

- Loop Partitioning: Dividing the loop into two or more separate loops may allow for each of those loops to be executed in parallel. This will increase the performance and the area.

- Memory Partitioning: The storage of the arrays in memory plays an important role in area and performance. On one hand, you could put an array entirely in one memory. And this memory can have a different number of ports (e.g., one or two ports for FPGA block RAM). Or you can divide the array into two or more memories to increase the number of ports. Or you could instantiate each of the variables as its own register, which allows simulataneous access to all of the variables at every clock cycle, but has high area costs.

# Submission Procedure

You should submit a report describing each of the architecture that you generated. This report should plot the results – e.g., resource utilization in terms of BRAMs, DSP48, FFs, LUTs, etc., and performance in terms of number of throughput (number of FIR operations/second), clock cycles, clock frequency (which is fixed to 10 ns). Some example graphs are below with results obtained using a variety of optimizations. Your report should describe each of the architectures that you generated. You should discuss the exact optimization and its resulting architecture. Figures describing the architecture are encouraged.

Figure 2. Area Results including the number of LUT, FF, DSP48 and BRAMs for seven different FIR architectures

Figure 3. Throughput Results  (Throughput = 1 /(Clock Perid (s)* #Clock Cycles))

The throughput is reported in Mhz using the formula from Equation 2. Note that you should use the "Estimated Clock Period (ns)" from HLS report instead of the specified clock period (which must be 10 ns).

The throughput in Hertz can be calculated as:

Throughput (Hz) = 1/(Clock Peridod (s)* #Clock Cycles).  			(Eq. 1)

The throughput in Mhz can be calculated as:

Throughput (Mhz) = [1/(Clock Peridod (s)* #Clock Cycles)]*1/106

=1000/(Clock Period(ns)*#Clock Cycles) 				(Eq. 2)

You must also submit your code (and only your code, not other files). Your code should have everything in it so that we can synthesize it directly. This means that you should use pragmas in your code, and not use the GUI to insert optimization directives. This means that we can only import your *.cpp file and it will directly synthesize. You can assume that we have correctly set up the design environment (fir_test.cpp, fir.h, etc.).

The folder should be zipped with a similar file structure as below:

FIR_lab_YOUR_LAST_NAME.zip

Contents:

- Report.pdf

- Folder “baseline”: original unmodified fir.cpp

- Folder “optimized1”: first optimization with fir.cpp

- Folder “optimized2”: second optimized design with fir.cpp

- …

- Folder “optimizedN”: nth optimized design with fir.cpp.

Email this to Janarbek Matai jmatai@cs.ucsd.edu with the title “CSE 237C Project 1”.