Project 2: Discrete Fourier Transform

# Introduction

The goal of this lab is design architectures that implement the Discrete Fourier Transform (DFT).  The DFT is a common operation in signal processing which generates a frequency domain representation of the discrete input signal. We start with the direct implementation of the DFT which is a matrix-vector multiplication. The input signal is a vector of samples and the matrix is composed of a set of basis functions corresponding to discrete cosine and sine waveforms of different frequencies. The multiplication of the input signal with these basis functions describes how well the input signal correlates with those waveforms, which is the value of the Fourier series at that frequency.

# Materials

You can find the following files in project2-dft.zip. These are divided into four folders dft_256, dft_256_precomputed, dft_1024, and dft_1024_precomputed. Each of these folders has its own out.gold.dat file since the output results are a little different depending on how you generate the DFT coefficients (and of course the 256 and 1024 results are different). You are only given one source file (dft.cpp), testbench (dft_test.cpp), and dft.h. You will have to change those files a bit in order to synthesize the different types of DFTs.

- dft.cpp  The baseline implementation for the dft function.

- dft.h header file

- dft_test.cpp  test bench

- table256.h  the constants for the real (cos_table) and imaginary (sin_table) 256 point DFT.

- table1024.h  the constants for the real (cos_table) and imaginary (sin_table) 1024 point DFT.

- out.gold.dat  “Golden” output. The testbench (dft_test.cpp) generates a sample input and calls the function dft in dft.cpp with that sample input. This output of the function is compared to the expected output. This will indicate PASS or FAIL. If it fails, then the code in dft is incorrect. There are four different versions of depending on the DFT size and way in which the DFT coefficients were generated.

- project2-dft.pdf  project instructions: this file

# Project Goal

You should modify the code to create a number of different architectures that tradeoff between performance and area. Your will create a report describing how you generated these different architectures (code restructuring, pragmas utilized, etc.). For each architecture you should provide its results including the resource utilization (BRAMs, DSP48, LUT, FF), and performance in terms of throughput (number of FFT operations/second), latency, clock cycles, clock frequency (which is fixed to 10 ns). You can do most of the design space exploration on the 256 point DFT. You should pick your “best” 256 architecture and synthesize that as a 1024 DFT.

# Optimization Hints and Guidelines

- You must always use a clock period of 10 ns.

- The output of your architecture must match the golden output.

- You should always use double for all data types. You do not need to perform bitwidth optimization of this project.

- The given code is unlikely to synthesize in any reasonable time with a 256 or 1024 DFT. It will synthesize with smaller size DFT (e.g., 8 or 16). This is due to the sin and cos function calls.

- There are many different ways to generate the DFT coefficients. These are constants when the DFT size is fixed. We have given you the coefficients for both 256 (in table256.h) and 1024 (in table1024.h). They each have two constant arrays, sin_table and cos_table. Your first optimization should be to use these tables to generate an architecture that will synthesize.

- There is significant amount of parallelism that can be exploited by (partially) unrolling the for loops. Pipelining these (partially) unrolled for loops should lead to higher throughputs.

- Your architecture must be able to perform any matrix-vector multiplication. In other words, you should not exploit any symmetry in the matrix coefficients. There are more efficient methods for performing the DFT that exploit the symmetries of the Fourier constants, e.g., the fast Fourier transform (FFT). Don’t worry, we will implement FFT architectures FFT soon enough.

# Submission Procedure

You must also submit your code (and only your code, not project files). Your code should have everything in it so that we can synthesize it directly. This means that you should use pragmas in your code, and not use the GUI to insert optimization directives. Each folder should have the necessary files to synthesize and test the code, e.g., dft_test.cpp, dft.h, etc.

The folder should be zipped with a similar file structure as below:

DFT_lab_YOUR_LAST_NAME.zip

Contents:

- “report.pdf” – A document describing all of your optimizations.

- Folder “baseline”: original unmodified dft.cpp

- Folder “architecture1”: the set of files required to synthesize your first architecture

- Folder “architecture2”: the set of files required to synthesize your second architecture

- …

- Folder “architectureN”: the set of files required to synthesize your Nth architecture

- Folder “architecture1024: the set of files required to synthesize your 1024 point DFT

Email this to Janarbek Matai <jmatai@cs.ucsd.edu> with the title “CSE 237C Project 2”.